RFID Communication Protocol V1.0

1. Product images

2. Connection diagram

3. Interface definition

RFID card reader connected to MDVR terminal

Baud rate: 115200, no checksum, data bit 8, stop bit 1

4. RFID card reader connected to PC

Baud rate: 115200, no checksum, data bit 8, stop bit 1

5. Data transmission protocol

Baud rate: 115200, no checksum, data bit 8, stop bit 1

Receive and send data format:

| Byte | 0 | 1 | 2 | 3 | N + 3 | N + 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Format | Frame header (0x7e) | Data type | Data length N |  | Data segment | Checksum |

Note: ① Checksum: frame beginning+data type+data length+data segment.

② Data type: Please refer to protocols 1, 2, 3, 4, etc. for details

- Read RFID card number protocol

Data type: 0x00

Data segment format when receiving commands:

Data length is 0

Data segment format when responding to commands:

Obtained card number

Example: Card number: 52 93 A0 02 53 01 04 E0

Sent: 7E 00 00 00 7E

Received: 7E 00 00 08 52 93 A0 02 53 01 04 E0 45

- Read RFID card data protocol

Data type: 0x01

Data segment format when receiving commands:

Byte0: Start block number

Byte1: Number of consecutive read blocks

Data segment format when responding to commands:

Obtained block data

Example: Read the 00th block, continuously read 8 blocks of data;

Sent: 7E 01 00 02 00 08 89

Received:7E 01 00 20 31 32 33 34  35 36 37 39 20 00 00 00 00 00 00 C5 4E 67 75 79 65 6E 20 56 61 6E 20 41 00 00 00 00 45

3. Write RFID card data protocol

Data type: 0x02

Data segment format when receiving commands:

Byte0 – Byte 14: Driver’s license number

Byte15: Check sum of the driver’s licentse number

Byte16-Byte59: Driver’s name

Byte60: Check sum of the driver’s name

Example: Write the Driver’s name is Nguyen Van a with the driver’s license number is 12345679:

Sent: 7E 02 00 3D 00 31 32 33 34 35 36 37 39 20 00 00 00 00 00 00 C5 4E 67 75 79 65 6E 20 56 61 6E 20 41 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 36 99

4、LED light and buzzer control protocol

Data type: 0x03

Data segment format:

Byte0：buzzer

Byte1：led_pwr

Byte2：led_net

Byte3：led_gps

Byte4：led_login

Byte5：led_rec

Byte6：led_alram

Byte7：led_cam0

Byte8：led_cam1

Byte9：led_cam2

Byte10：led_cam3

Byte_Value: The low 4 digit value (0: constant off, 1: constant on, 2: single flashing, 3: continuous flashing), when flashing, the high 4 digits indicate the flashing frequency, flashing several times in 1 second, and the maximum should not exceed 5

Example:

- full leds off:

Sent: 7e 03 00 0b 00 00 00 00 00 00 00 00 00 00 00 8C

No response

- full leds on;

Sent; 7e 03 00 0b 01 01 01 01 01 01 01 01 01 01 01 97

No response

5、Transparent data protocol

Data type: 0x04

Data segment format:

Data to be transmitted through

Sent: 7e 04 00 15 24 47 53 48 54 2c 31 2c 34 2c 3c 31 30 2c 30 3e 2c 31 36 23 00 c7

Received: 7e ff 00 01 01 7f//indicates successful transparent transmission with data type ff